System-Synchronous Input (Direct or MMCM-based Clock)

| Input, System Synchronous, SDR, Rising Edge |  |
| --- | --- |
|  | Clock = 10 ns Trco_max = 3.1 ns Trco_min = 1.9 ns trce_dly = 0 Equivalent using data valid: dv_bre = 6.9 ns dv_are = 1.9 ns |
| create_clock -period 10.000 -name clkin_sys [get_ports clkin_sys] set_input_delay -clock [get_clocks clkin_sys] -min -add_delay 1.900 [get_ports din_sys_re] set_input_delay -clock [get_clocks clkin_sys] -max -add_delay 3.100 [get_ports din_sys_re] |  |
| report_timing -from [get_ports din_sys_re] -setup report_timing -from [get_ports din_sys_re] -hold |  |
| Notes: Using an MMCM for the system-synchronous clocks helps reduce the clock delay, and thus provides addional hold slack. It also helps to minimize the clock-to-out delay for system-synchronous outputs. The overall system clock can become faster as PVT variations in the clock path will be minimized. |  |

| Input, System Synchronous, SDR, Rising Edge |  |
| --- | --- |
| Setup Timing Analysis, Rising Edge Capture | Hold Timing Analysis, Rising Edge Capture |

System-Synchronous Input (Direct or MMCM-based Clock)

| Input, System Synchronous, SDR, Falling Edge |  |
| --- | --- |
|  | Clock = 10 ns Tfco_max = 3 ns Tfco_min = 1.8 ns trce_dly = 0 Equivalent using data valid: dv_bfe = 7 ns dv_afe = 1.8 ns |
| create_clock -period 10.000 -name clkin_sys [get_ports clkin_sys] set_input_delay -clock [get_clocks clkin_sys] -clock_fall -min -add_delay 1.800 [get_ports din_sys_fe] set_input_delay -clock [get_clocks clkin_sys] -clock_fall -max -add_delay 3.000 [get_ports din_sys_fe] |  |
| report_timing -from [get_ports din_sys_fe] -setup report_timing -from [get_ports din_sys_fe] -hold |  |
| Notes: Using an MMCM for the system-synchronous clocks helps reduce the clock delay, and thus provides addional hold slack. It also helps to minimize the clock-to-out delay for system-synchronous outputs. The overall system clock can become faster as PVT variations in the clock path will be minimized. |  |

| Input, System Synchronous, SDR, Falling Edge |  |
| --- | --- |
| Setup Timing Analysis, Falling Edge Capture | Hold Timing Analysis, Falling Edge Capture |

System-Synchronous Input (Direct or MMCM-based Clock)

| Input, System Synchronous, DDR |  |
| --- | --- |
|  | Clock = 10 ns (5 ns/ 5 ns) Trco_max = 1.9 ns Trco_min = 1.3 ns Tfco_max = 2 ns Tfco_min = 1.4 ns trce_dly = 0 Equivalent using data valid: dv_bfe = 3.1 ns dv_afe = 1.3 ns dv_bre = 3.0 ns dv_are = 1.4 ns |
| create_clock -period 10.000 -name clkin_sys [get_ports clkin_sys] set_input_delay -clock [get_clocks clkin_sys] -clock_fall -min -add_delay 1.400 [get_ports din_sys_ddr] set_input_delay -clock [get_clocks clkin_sys] -clock_fall -max -add_delay 2.000 [get_ports din_sys_ddr] set_input_delay -clock [get_clocks clkin_sys] -min -add_delay 1.300 [get_ports din_sys_ddr] set_input_delay -clock [get_clocks clkin_sys] -max -add_delay 1.900 [get_ports din_sys_ddr] |  |
| report_timing -from [get_ports din_sys_ddr] -fall_to [get_clocks] –setup report_timing -from [get_ports din_sys_ddr] -fall_to [get_clocks] –hold report_timing -from [get_ports din_sys_ddr] -rise_to [get_clocks] –setup report_timing -from [get_ports din_sys_ddr] -rise_to [get_clocks] -hold |  |
| Notes: Using an MMCM for the system-synchronous clocks helps reduce the clock delay, and thus provides addional hold slack. It also helps to minimize the clock-to-out delay for system-synchronous outputs. The overall system clock can become faster as PVT variations in the clock path will be minimized. |  |

| Input, System Synchronous, DDR |  |
| --- | --- |
| Setup Timing Analysis, Falling Edge Capture | Hold Timing Analysis, Falling Edge Capture |
| Setup Timing Analysis, Rising Edge Capture | Hold Timing Analysis, Rising Edge Capture |

System-Synchronous Output Timing Specification (Setup/Hold Based)

| Setup/Hold Based, SDR, Rising Edge (System Synchronous) |  |
| --- | --- |
|  | Clock = 10 ns tsu_r = 1.1 ns thd_r= 0.9 ns trc_dly = 0 |
| create_clock -period 10.000 -name clkin_sys [get_ports clkin_sys] set_output_delay -clock [get_clocks clkin_sys] -min -add_delay -0.900 [get_ports dout_sys_re] set_output_delay -clock [get_clocks clkin_sys] -max -add_delay 1.100 [get_ports dout_sys_re] |  |
| report_timing -to [get_ports dout_sys_re] -setup report_timing -to [get_ports dout_sys_re] -hold |  |

| Setup/Hold Based, SDR, Rising Edge (System Synchronous) |  |
| --- | --- |
| Setup Analysis, Rising Capture Edge | Hold Analysis, Rising Capture Edge |

System-Synchronous Output Timing Specification (Setup/Hold Based)

| Setup/Hold Based, SDR, Falling Edge (System Synchronous) |  |
| --- | --- |
|  | Clock = 10 ns tsu_f = 1.0 ns thd_f= 0.8 ns trc_dly = 0 |
| create_clock -period 10.000 -name clkin_sys [get_ports clkin_sys] set_output_delay -clock [get_clocks clkin_sys] -clock_fall -min -add_delay -0.800 [get_ports dout_sys_fe] set_output_delay -clock [get_clocks clkin_sys] -clock_fall -max -add_delay 1.000 [get_ports dout_sys_fe] |  |
| report_timing -to [get_ports dout_sys_fe] -setup report_timing -to [get_ports dout_sys_fe] -hold |  |

| Setup/Hold Based, SDR, Falling Edge (System Synchronous) |  |
| --- | --- |
| Setup Analysis, Falling Capture Edge | Hold Analysis, Falling Capture Edge |

System-Synchronous Output Timing Specification (Setup/Hold Based)

| Setup/Hold Based, DDR (System Synchronous) |  |
| --- | --- |
|  | Clock = 10 ns tsu_r = 0.4 ns thd_r= 0.2 ns tsu_f = 0.5 ns thd_f= 0.3 ns trc_dly = 0 |
| create_clock -period 10.000 -name clkin_sys [get_ports clkin_sys] set_output_delay -clock [get_clocks clkin_sys] -clock_fall -min -add_delay -0.300 [get_ports dout_sys_ddr] set_output_delay -clock [get_clocks clkin_sys] -clock_fall -max -add_delay 0.500 [get_ports dout_sys_ddr] set_output_delay -clock [get_clocks clkin_sys] -min -add_delay -0.200 [get_ports dout_sys_ddr] set_output_delay -clock [get_clocks clkin_sys] -max -add_delay 0.400 [get_ports dout_sys_ddr] |  |
| report_timing -to [get_ports dout_sys_ddr] –rise_from [get_clocks] -setup report_timing -to [get_ports dout_sys_ddr] –fall_from [get_clocks] -hold report_timing -to [get_ports dout_sys_ddr] –fall_from [get_clocks] -setup report_timing -to [get_ports dout_sys_ddr] –rise_from [get_clocks] -hold |  |

| Setup/Hold Based, DDR (System Synchronous) |  |
| --- | --- |
| Setup Analysis, Falling Capture Edge | Hold Analysis, Falling Capture Edge |
| Setup Analysis, Rising Capture Edge | Hold Analysis, Rising Capture Edge |